Problem 1:

| Area: 520.786 | Cell Counts: 139 | Timing Slack: 12 |
| --- | --- | --- |

Report Area:

Report Timing:

RTL:
`timescale 1ns/1ps

module mac(

input signed [3:0] IN,W, //should be signed

input clk, rstb,

output reg signed [11:0] OUT //should be signed

);

reg signed [6:0] A; //should be signed

reg signed [10:0] B; //should be signed

reg [3:0] counter;

always @(posedge clk or negedge rstb) begin

if(~rstb) begin

counter = 0;

OUT = 0;

B = 0;

A = 0;

end else if(counter == 9) begin

counter = 0;

OUT = B;

B = 0;

A = IN*W;

end else if(counter < 9) begin

counter = counter + 1;

A = IN*W;

B = A+B;

end else begin

counter = 0;

OUT = 0;

B = 0;

A = 0;

end

end

endmodule

Testbench:
`timescale 1ns/1ps

module mac_tb;

reg signed [3:0] IN,W; //should be signed

reg clk, rstb;

wire signed [11:0] OUT; //should be signed

mac mymac(IN,W,clk,rstb,OUT);

always begin

$dumpfile("mac.vcd");

$dumpvars(0,mac_tb);

IN = 0;

W = 0;

rstb = 1;

clk = 1;

repeat(21) begin

#0.5

clk = 0;

#0.5

clk = 1;

IN = IN + 1;

W = W + 2;

end

$finish;

end

endmodule

SDC:

create_clock -name clk -period 1.0 -waveform { 0 0.5 } [get_ports clk]

set_input_delay -max 0.5 -clock clk [get_ports {IN W}]

set_input_delay -min -0.2 -clock clk [get_ports {IN W rstb}]

set_output_delay -max 0.5 -clock clk [get_ports {OUT}]

set_output_delay -min -0.2 -clock clk [get_ports {OUT}]

Problem 3)

Part d. Verification of FSM

FSM code:

`timescale 10ns/1ps

module fsm(

input bi,clk,

output reg bo

);

reg q0,q1;

always @(posedge clk) begin

q1 = q0 & bi;

q0 = bi;

bo = ~q1 & q0;

end

endmodule